PATHFINDER BEGINNER - NINTENDO SWITCH   CODING CHEAT SHEET

The C++ you need  (1 of 2)

Everything you need to write the code in this course. Stuck? Look here first, then ask your group, then an instructor.

1   The shape of every program

| INITIALIZATION: the top of the file. Libraries, settings and variables. Read once, before anything runs. | #include <Adafruit_NeoPixel.h> // pull in a library const int LED_PIN = 2; // a setting: never changes int loopCount = 0; // a variable: can change |
| --- | --- |
| setup() runs ONCE, at power-up or reset. Get the hardware ready. | void setup() { pinMode(LED_PIN, OUTPUT); Serial.begin(115200); } |
| loop() runs over and over, as fast as it can, until the power goes off. | void loop() { loopCount = loopCount + 1; digitalWrite(LED_PIN, HIGH); delay(1000); } |

The rules that catch everybody.  Every statement ends with  ;.  Every  {  has a matching  }.  //  starts a comment the computer ignores.  Capitals matter:  Serial  is not  serial.  A name is one word - letters, digits and _ - and cannot start with a digit.

2   Variables: named boxes that hold a value

| Type | Holds | Example |
| --- | --- | --- |
| int | A whole number, up to about 2 billion either way | int speed = 200; |
| float | A number with a decimal point | float volts = 16.8; |
| bool | Only true or false | bool lightsOn = true; |
| unsigned long | A big whole number, never negative - for millis() | unsigned long lastBlink = 0; |
| uint32_t | A NeoPixel color, packed into one number | uint32_t red = strip.Color(255, 0, 0); |
| const | In front of any type: it can never change | const int MOTOR_MAX = 255; |
| uint8_t x[6] | A list of six numbers, 0 to 255 | const uint8_t MY_CONTROLLER[6] = { 0x58, 0x2F, 0x40, 0x11, 0x22, 0x33 }; |

Make a variable ONCE, with its type; after that use the name on its own:  speed = 150;  A variable made inside  { }  disappears at the  }. If loop() must remember it from one pass to the next, make it at the top of the file.

3   Math, and the traps in it

| Arithmetic. Whole numbers give whole answers: the fraction is thrown away, never rounded. | int total = 7 + 2; // 9 int half = 7 / 2; // 3, NOT 3.5 float exact = 7.0 / 2; // 3.5 - one decimal point does it int leftOver = 7 % 2; // 1, the remainder int none = 9 / 10; // 0, and it does not round up |
| --- | --- |
| Changing a variable | count = count + 1; count++; // both add one count += 5; count--; // add five, take one away |
| Multiply BEFORE you divide, or it comes out 0. | int percent = duty * 100 / 255; // duty 128 gives 50 int wrong = duty / 255 * 100; // duty 128 gives 0 |
| Handy functions | int size = abs(-40); // 40: no sign int safe = constrain(300, -255, 255); // 255: kept in range int speed = map(255, 0, 511, 0, 255); // 127: proportion int small = min(3, 8); // 3 (and max() gives the larger) int pick = random(0, 32); // 0 to 31: the top is left out |

4   Comparing, and combining tests

| Comparisons come out true or false. TWO = signs to compare: one = SETS the value instead. | a == b equal a != b not equal a < b less than a <= b less than or equal a > b more than a >= b more than or equal if (speed = 0) // WRONG: sets speed to 0, never compares |
| --- | --- |
| Combine tests: AND, OR, NOT. | if (speed > 0 && lightsOn) { } // && both must be true if (stickX < -20 ¦¦ stickX > 20) { } // ¦¦ either one will do if (!lightsOn) { } // ! flips true and false |

PATHFINDER BEGINNER - NINTENDO SWITCH   CODING CHEAT SHEET

The C++ you need  (2 of 2)

5   Making decisions: if, else if, else

| if runs its block only when the test is true; else when it is not - never both. else if makes a chain: the FIRST true test wins, so put the narrowest test first. test ? a : b is a quick either/or for one value: a if the test is true, b if not. | if (duty > 200) { Serial.println("fast"); } else if (duty > 60) { Serial.println("moving"); } else { Serial.println("stopped"); } int forwardDuty = (speed > 0) ? speed : 0; Serial.println(lightsOn ? "Lights ON" : "Lights OFF"); |
| --- | --- |

6   Loops: doing something many times

| for a known number of times: start here; keep going while this is true; change this each time round. | for (int i = 0; i < 32; i++) { strip.setPixelColor(i, strip.Color(0, 0, 255)); } for (int i = 15; i >= 0; i--) { strip.setPixelColor(i, 0); } // down for (int i = 0; i < 32; i += 2) { strip.setPixelColor(i, 0); } // in twos |
| --- | --- |
| A run of LEDs: use <= when the last number counts, or you miss it - off by one. | for (int i = FRONT_FIRST; i <= FRONT_LAST; i++) { strip.setPixelColor(i, strip.Color(255, 255, 255)); } |
| while goes on as long as the test stays true, and might not run at all. break; leaves any loop early. | while (countdown > 0) { // countdown is an int countdown--; } |

7   Functions: give a job a name, then call it

| void means it hands nothing back. Parameters pass information in. Write it outside every other function, then call it from loop(). | void lightRange(int first, int last, uint32_t color) { for (int i = first; i <= last; i++) { strip.setPixelColor(i, color); } } lightRange(0, 15, strip.Color(255, 0, 0)); // call it: front bar red |
| --- | --- |
| return hands a value back: its type goes where void was. | int percentOf(int duty) { return duty * 100 / 255; // percentOf(128) is 50 } |

8   The Serial Monitor: how the program talks to you

| Tools > Serial Monitor. Nonsense characters: the baud box is not 115200. Stuck? Print the numbers you are working from. | Serial.begin(115200); // in setup() Serial.print("Speed: "); // print stays on the line Serial.println(speed); // println ends it Serial.println(volts, 2); // two decimal places Serial.print("\t"); // a tab, to line up columns |
| --- | --- |

9   Time: delay() stops everything, millis() does not

| delay(1000) waits a second and NOTHING else happens, so never in a vehicle program. millis() is how long the board has been on, in ms. Check it every pass; act when enough time has gone by. Every job gets its OWN last-time variable. | unsigned long lastBlink = 0; void loop() { if (millis() - lastBlink >= 500) { // every half second lastBlink = millis(); blinkOn = !blinkOn; // ...do the job... } // everything else keeps running } |
| --- | --- |

10   Named states, lists, and reading sensors

| enum names the states something can be in. An array is a list: [i] picks one, counting from 0. Sensors for the projects are read like the last two lines. | enum LightPattern { LIGHTS_STOPPED, LIGHTS_FORWARD, LIGHTS_REVERSE }; LightPattern pattern = LIGHTS_STOPPED; // one named state const int CORNERS[] = { 0, 15, 16, 31 }; // CORNERS[0] is 0 const int HOW_MANY = sizeof(CORNERS) / sizeof(CORNERS[0]); // 4 int millivolts = analogReadMilliVolts(34); // 0 to about 3300 long echo = pulseIn(ECHO_PIN, HIGH, 30000); // microseconds, 0 if none |
| --- | --- |

PATHFINDER BEGINNER - NINTENDO SWITCH   CODING CHEAT SHEET

This vehicle  (1 of 2)

11   The pins this vehicle uses

| What | GPIO pins | Notes |
| --- | --- | --- |
| Left motors | front 12, 13 rear 18, 19 | Two pins each: the first drives forward, |
| Right motors | front 22, 23 rear 16, 17 | the second reverse |
| The 32 LEDs | 5 | One data wire drives all of them |
| Servos 1 to 4 | 25, 26, 27, 14 | On the main board. Breadboard LED: 2 |
| I2C sensors | SDA 32, SCL 33 | GPIO 34 to 39 are INPUT only |

12   Motors

| Set each pin up once, in setup(). | ledcSetup(FRONT_LEFT_CH_A, 20000, 8); // channel, 20 kHz, 8 bits ledcAttachPin(FRONT_LEFT_PIN_A, FRONT_LEFT_CH_A); |
| --- | --- |
| Duty 0 to 255 is the power. A for forward, B for reverse; both 0 coasts. | ledcWrite(FRONT_LEFT_CH_A, 200); // forward, about 78% ledcWrite(FRONT_LEFT_CH_B, 0); // B off |
| drive(left, right) does all four motors: -255 to +255, minus is reverse. | drive(200, 200); // straight ahead drive(150, -150); // spin right on the spot drive(0, 0); // stop |
| Tank drive: one stick, two sides. constrain() stops 200 + 60 asking for 260. | int leftSpeed = constrain(forward + turn, -MOTOR_MAX, MOTOR_MAX); int rightSpeed = constrain(forward - turn, -MOTOR_MAX, MOTOR_MAX); drive(leftSpeed, rightSpeed); |

A wheel turns the wrong way? Swap that motor's two pin numbers in the sketch - never rewire.  Wheels off the ground the first time you upload any change that drives.

13   The 32 LEDs  (Adafruit NeoPixel)

| Make the strip once, at the top of the file. | #include <Adafruit_NeoPixel.h> const int LED_PIN = 5; const int LED_COUNT = 32; Adafruit_NeoPixel strip(LED_COUNT, LED_PIN, NEO_GRB + NEO_KHZ800); |
| --- | --- |
| In setup() | strip.begin(); strip.setBrightness(60); // 0 to 255, set once |
| Paint, then show. These only change memory: NOTHING lights up until strip.show(). | strip.setPixelColor(4, strip.Color(255, 0, 0)); // one LED strip.fill(strip.Color(0, 255, 0)); // all of them strip.clear(); // all off strip.show(); // now send it! |
| Rainbows use a HUE, 0 to 65535, round the color wheel. | strip.setPixelColor(0, strip.gamma32(strip.ColorHSV(21845))); int howMany = strip.numPixels(); // 32 |

The LED map. Front bar 0 (left) to 15 (right); the rear bar runs back, 16 (right) to 31 (left). The LED behind front LED p is  31 - p.  LEFT side: 0-7 and 24-31.  RIGHT side: 8-15 and 16-23.  Corners: 0, 15, 16, 31.

|  | LEFT side of the vehicle |  |  |  |  |  |  |  | RIGHT side of the vehicle |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FRONT | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| REAR | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |

Colors:  each one is  strip.Color(red, green, blue)

| Red | Color(255, 0, 0) | Yellow | Color(255, 255, 0) | Purple | Color(128, 0, 255) |
| --- | --- | --- | --- | --- | --- |
| Green | Color(0, 255, 0) | Cyan | Color(0, 255, 255) | White | Color(255, 255, 255) |
| Blue | Color(0, 0, 255) | Magenta | Color(255, 0, 255) | Warm white | Color(255, 160, 80) |
| Orange | Color(255, 100, 0) | Pink | Color(255, 100, 100) | Off | Color(0, 0, 0) |

Power: each LED at full white draws about 60 mA, so all 32 draw nearly 2 amps. Red lights one of the three LEDs inside a pixel; white lights all three, and costs three times as much.

PATHFINDER BEGINNER - NINTENDO SWITCH   CODING CHEAT SHEET

This vehicle  (2 of 2)

14   The controller

| At the top: the library, and YOUR controller's address - paste the line l3a_controller_check prints. | #include <Bluepad32.h> const uint8_t MY_CONTROLLER[6] = { 0x00, 0x00, 0x00, 0x00, 0x00, 0x00 }; ControllerPtr myController = nullptr; |
| --- | --- |
| Setting up takes two callbacks and an allowlist: copy them from l3c_tank_drive. | BP32.update(); // the first line of loop(), every time |
| Sticks: -512 to +511. Pushing UP gives a NEGATIVE number. | int leftX = myController->axisX(); // left and right int leftY = myController->axisY(); // UP is negative int rightX = myController->axisRX(); // axisRY() for up and down |
| Buttons are named by POSITION, not by the letter printed on the pad. True while held down. | a() bottom (B) b() right (A) x() left (Y) y() top (X) l1() r1() l2() r2() shoulders and triggers thumbL() thumbR() click a stick in bool upPressed = myController->dpad() & DPAD_UP; |

15   The patterns the full program is built from

| FAILSAFE. No controller, no driving: the very first lines of loop(). | if (myController == nullptr ¦¦ !myController->isConnected()) { drive(0, 0); return; // skip the rest of loop() } |
| --- | --- |
| DEADZONE AND MAP. Nearly centered counts as centered; the rest of the stick is stretched across the whole speed range. | int stickToSpeed(int stickValue, int maxSpeed) { if (abs(stickValue) < STICK_DEADZONE) { return 0; } int size = map(abs(stickValue), STICK_DEADZONE, STICK_MAX, MOTOR_MIN, maxSpeed); size = constrain(size, MOTOR_MIN, maxSpeed); return (stickValue > 0) ? size : -size; } |
| EDGE DETECTION. One press, one action, however long the finger stays down. One wasDown per button. | bool justPressed(bool isDown, bool &wasDown) { bool isNewPress = isDown && !wasDown; wasDown = isDown; return isNewPress; } |
| TOGGLE on each new press, and DIRTY FLAG: redraw only when something changed. | if (justPressed(myController->x(), lightsWasDown)) { lightsOn = !lightsOn; lightsChanged = true; } if (lightsChanged) { lightsChanged = false; showDrivingLights(); } |

16   When it will not compile, or will not work

| What you see | What it usually means |
| --- | --- |
| 'speed' was not declared in this scope | Spelled differently (capitals count), made inside other { }, or a missing #include. |
| expected ';' before ... | A missing ; - usually at the end of the line ABOVE. |
| expected '}' at end of input | A { with no matching }. |
| Bluepad32.h: No such file or directory | Wrong board: pick the esp32_bluepad32 ESP32 Dev Module. |
| MY_CONTROLLER has not been filled in | Run l3a_controller_check and paste the line it prints. |
| The controller will not connect | MY_CONTROLLER is not this pad. Is it charged? Hold SYNC. |
| Failed to connect to ESP32 | Charge-only cable, wrong port, or the vehicle is off. |
| Serial Monitor shows nonsense | The baud box is not 115200. |
| The LEDs do not change | No strip.show(). |
| It creeps on its own | STICK_DEADZONE is too small. |